Chapter 2 Lab Solutions

Activity Questions Lab 2.1

There are no activity questions in this lab.

Review Questions Lab 2.1

  - Which of the devices that you used in this lab are considered LAN equipment?
Bridges, hubs, and switches are LAN equipment.

  - Which of the devices that you used in this lab are considered WAN equipment?
Routers and CSU/DSUs are considered WAN equipment.

  - In Figure 2-1, are computers A, B, and C on the same network as D, E, and F?
Yes, because they are all off the same interface on the router.

  - In Figure 2-1, are computers A, B, and C on the same network as G, H, and I?
No, because they are off different interfaces on the router.

  - How does the bridge operate to filter traffic between the two attached segments in your network?
The bridge learns which MAC addresses are associated with which segments by looking at frames that come into the bridge and by building a table. This bridging table maps the segments to associated MAC addresses. The bridge uses the table when deciding whether a frame will be forwarded to a different segment. If there is no reference to a MAC address in the table, the bridge forwards the frame out every port except the one it came in on.

  - In Figure 2-1, how does the router operate to filter traffic between the segments off of E0, E1, and S0?
The router learns about networks and their locations either from a network administrator typing in the information (statically) or from dynamic routing protocols. This Network layer addressing information (logical addresses) is stored in a routing table. When the router receives a packet, it looks in its routing table to decide where to forward the packet.

  - What kind of domains do bridges and switches create?
Collision domains.

  - What kind of domains do routers create?
Collision and broadcast domains.

  - If the LANs in Figure 2-1 are 100-Mbps systems, which computers are sharing this bandwidth?
A, B, C, D, E, and F are sharing 100 Mbps but G, H, and I are on a switch so they have their own 100 Mbps.

  - What kind of traffic is a bridge and switch unable to filter?
Broadcast traffic.

Activity Questions Lab 2.2

| Characteristic | Device(s) |
| --- | --- |
| Operates at upper layers to translate between different protocol suites | -gateway |
| Filters traffic based on MAC address | -bridge -switch |
| Introduces the most latency on a network | -gateway |
| Boosts the signal but does not segment the network | -hub -repeater |
| Operates differently depending on whether nonroutable or routable protocols are in use | -brouter |
| Creates broadcast domains | -router |
| Creates a virtual circuit between sender and receiver | -switch |
| Forwards broadcast traffic | -bridge -switch -hub and repeater |
| Filters traffic based on logical address | -router -brouter |
| Associated with the term “microsegmentation” | -switch |
| Creates subnetworks | -router |
| Connects computers in a physical star and uses “shared bandwidth” | -hub |
| Creates collision domains | -bridge -switch -router -brouter |
| Operates at layer 1 of the OSI model | -hub |
| Operates at layer 2 of the OSI model | -bridge -switch -brouter |
| Operates at layer 3 of the OSI model | -brouter -router |

Review Questions Lab 2.2

- When is it appropriate to introduce a router into your network?
When you want to create additional networks and/or broadcast domains.

- What can bridges do that hubs and repeaters cannot do?
Bridges can segment the network and create collision domains.

- When would introducing a brouter on your network be appropriate?
Introduce a brouter when you are using nonroutable as well as routable protocols on your network.

- What are the advantages and disadvantages of using a gateway on your network?
A gateway allows you to use multiple protocol suites on your network and communicate between them, but it does introduce latency.

- What is the benefit of replacing hubs on your network with switches?
Switched bandwidth provided by switches offers significantly better performance than the shared bandwidth provided by hubs.

Activity Questions Lab 2.3

2.

| # Segment | # Address |
| --- | --- |
| 1 | MAC of A, B, C, and D |
| 2 | MAC of E, F, G, and H |
| 3 | MAC of I, J, K, and L |

4.

| # Port | # Address |
| --- | --- |
| 1 | MAC of A |
| 2 | MAC of B |
| 3 | MAC of C |
| 4 | MAC of D |
| 5 | MAC of E |
| 6 | MAC of F |
| 7 | MAC of G |
| 8 | MAC of H |
| 9 | MAC of I, J, K, and L |

Review Questions Lab 2.3

- How many collision domains are defined in Figure 2-2?
Three collision domains are defined—one for each segment of the bridge.

- How many collision domains are defined in Figure 2-3?
Whenever hosts communicate on a switch using unicast traffic, a collision domain will exist between those two hosts only. So, there are nine total collision domains. All four hosts on the hub are sharing one collision domain.

- Examine Figure 2-2. If a frame from computer A has a destination MAC address for computer D, which devices will see the layer 2 information?
A, B, C, D, and the bridge will see the layer 2 information.

- Examine Figure 2-3. If a frame from computer A has a destination MAC address for computer D, which devices will see the layer 2 information?
Only A and D and the switch will see the layer 2 information.

- Examine Figure 2-3. In terms of network performance, what is the difference between computers A through H and I through L?
A through H are able to take advantage of the switched bandwidth provided by the switch. I through L have to share bandwidth because they are attached to a hub, which is then attached to a switch port.

- What will a bridge and switch do with a frame for which it has no information about the destination in its table?
The switch and bridge will forward the frame out all ports except the one it came in on.

- How do bridges and switches handle broadcast frames?
Bridges and switches forward broadcast frames.

- Why is a switch considered more effective than a bridge in terms of increasing network performance?
Because a switch microsegments by creating a virtual circuit between the sending node and the receiving node for unicast traffic. These two nodes get the entire network segment bandwidth and are not subject to collisions between other nodes on the network segment.

- In general, how do bridges and switches dynamically create their tables?
They learn which MAC addresses are associated with which ports by examining frames that come across their interfaces.

- What happens to tables when bridges and switches are turned off?
Dynamic tables are dumped out of memory and have to be rebuilt again when the bridge or switch is turned back on. Tables that were built statically by the network administrator remain.

Activity Questions Lab 2.4

1. Organization that developed the 802.11 specifications IEEE

2. Wireless security standard that replaced WEP and uses the TKIP algorithm WPA

3. The latest and fastest 802.11 standard 802.11n

4. This mode specifies that the client device associates with an access point infrastructure mode

5. The wireless network name that is often broadcast so that clients can associate using it SSID

6. The IEEE port-blocking specification 802.1x

7. The network access method used by 802.11 devices CSMA/CA

8. The IEEE security standard based on WPA2 802.11i

9. The IEEE standard that specifies 54 Mbps in the 5 GHz range 802.11a

10. This mode specifies that the client device communicate directly with other client devices and that no access point is used ad hoc mode

11. A single access point in infrastructure mode BSS

12. The original security standard for 802.11 WEP

13. The IEEE standard that specifies 11 Mbps in the 2.4 GHz range 802.11b

14. Multiple access points connected in infrastructure mode ESS

15. The IEEE standard that specifies 54 Mbps in the 2.4 GHz range 802.11g

16. Organization developed to promote 802.11 usage Wi-Fi Alliance

Review Questions Lab 2.4

- What is the greatest advantage of using wireless?
Mobility

- What is the greatest disadvantage of using wireless?
Security

- In terms of bandwidth, is an access point more like a switch or a hub? Explain.
A hub because the bandwidth is shared between all users associated with it

- Which is more efficient and why? CSMA/CD or CSMA/CA?
CSMA/CD because with CSMA/CA all frames must be individually acknowledged by the receiver. Wireless devices cannot detect collisions

- How does a wireless client know which access point to associate with?
It scans for a broadcasted network name (SSID) or it probes for a particular SSID. If there is more than one access point to associate with, the client will choose the one with the strongest signal and lowest error rate.